TECHNICAL BULLETIN

FOR KS TELECOM DEALERS AND DISTRIBUTORS

REF:B6070901

PRODUCT: KSX-64 & KSX 256 CMU CARD VER: 93.01

SUBJECT: LOSS OF SPECIAL TONE ON SLT PORTS.

A problem with losing ICM dial tone to SLT ports with MSG waiting lights activated may occur.

If this problem occurs please reverse components C8 and C9 using the illustration below.

C8 should be the same value as C4 and C17.

C9 should be the same value as C2 and C14.