TECHNICAL BULLETIN

FOR KS TELECOM DEALERS AND DISTRIBUTORS

REF: 6080202

PRODUCT: SALTA 616

SUBJECT: REPLACING SOFTWARE

1. Power system OFF.

2. Remove KSU cover and locate the CCU CARD.

3. Carefully remove both chips from the sockets marked U12 and U17.

4. Replace with new chips. Be sure to place chips in the correct socket with the             “notched” end of the chip matching the “notched” end of the chip socket. Verify   that all     pins of the chip fit securely into the chip socket and do not fold under       the chip or fall  outside the chip socket.

5. Turn the battery switch (SW1) to the “OFF” position.

6. Power the system ON and re-program.

7. Turn the battery switch (SW1) to the “ON” position.